MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

OBJECTIVES:

i) To model and simulate the operation of a superheterodyne receiver for an AM communication system using MATLAB by implementing frequency mixing and intermediate frequency (IF) signal generation.

ii) To recover the original message signal through intermediate frequency filtering and envelope detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

PROGRAM (MATLAB CODE):

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

% Frequencies

fc = 10000;       % Carrier frequency

fm = 400;         % Message frequency

fi = 5000;        % Intermediate frequency

flo = fc + fi;    % Local oscillator frequency

% 1. Message Signal

m = cos(2*pi*fm*t - pi/3);

% 2. Carrier Signal

c = cos(2*pi*fc*t);

% 3. AM Signal

rf = (1 + m).*cos(2*pi*fc*t);

% 4. Mixer and IF Signal

lo = cos(2*pi*flo*t);

mixed = rf .* lo;

% Approximate IF signal for display

if_signal = 0.5*(1 + m).*cos(2*pi*fi*t);

% 5. Demodulation

demod = if_signal .* 2.*cos(2*pi*fi*t);

% Low-pass filtering using moving average

output = movmean(demod, 100);

% Remove DC component to recover message

output = output - mean(output);

% Normalize recovered signal

output = output / max(abs(output));

% Plot all signals

figure('Color','w');

subplot(5,1,1);

plot(t,m,'b','LineWidth',1);

title('Message Signal');

xlim([0 0.02]);

ylim([-1.1 1.1]);

grid on;

subplot(5,1,2);

plot(t,c,'b','LineWidth',1);

title('Carrier Signal');

xlim([0 0.02]);

ylim([-1.1 1.1]);

grid on;

subplot(5,1,3);

plot(t,rf,'b','LineWidth',1);

title('AM Signal');

xlim([0 0.02]);

ylim([-2 2]);

grid on;

subplot(5,1,4);

plot(t,if_signal,'b','LineWidth',1);

title('Intermediate Frequency (IF) Signal');

xlim([0 0.02]);

ylim([-1.1 1.1]);

grid on;

subplot(5,1,5);

plot(t,output,'b','LineWidth',1);

title('Recovered Signal');

xlabel('Time (s)');

xlim([0 0.02]);

ylim([-1.1 1.1]);

grid on;

% Display frequency values

fprintf('RF Frequency = %d Hz\n',fc);

fprintf('Message Frequency = %d Hz\n',fm);

fprintf('IF Frequency = %d Hz\n',fi);

fprintf('Local Oscillator Frequency = %d Hz\n',flo);

OUTPUT (GRAPH)

RESULT:

The superheterodyne receiver was successfully simulated in MATLAB, and the message signal, carrier signal, AM signal, Intermediate Frequency (IF) signal, and recovered signal were obtained through frequency mixing and filtering.