GENERATION AND ANALYSIS OF FREQUENCY MODULATED

(FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

OBJECTIVES:

To generate and analyze Frequency Modulated (FM) and Phase Modulated (PM) signals using

MATLAB by varying the instantaneous frequency and phase of a carrier signal in accordance

with the message signal.

PROGRAM (MATLAB CODE):

clc;

clear;

close all;

Am=1;

Ac=1;

fm=5;

fc=50;

beta=5;

kp=5;

fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t); % Message signal

c = Ac*cos(2*pi*fc*t); % Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM

pm_sig = Ac*cos(2*pi*fc*t + kp*m); % PM

Df = beta*fm; % Frequency deviation

fprintf('Frequency Deviation = %d Hz\n',Df);

fprintf('FM Modulation Index = %d\n',beta);

subplot(4,1,1), plot(t,m), title('Message Signal')

subplot(4,1,2), plot(t,c), title('Carrier Signal')

subplot(4,1,3), plot(t,fm_sig), title('FM Signal')

subplot(4,1,4), plot(t,pm_sig), title('PM Signal')

OUTPUT GRAPH:

RESULT:

The Frequency Modulated (FM) and Phase Modulated (PM) signals were successfully generated using MATLAB by varying the instantaneous frequency and phase of the carrier signal according to the message signal.

• Frequency Deviation (Δf) = 25 Hz

• FM Modulation Index (β) = 5